Bay of Plenty Marine Oil Spill Contingency Plan

(Tier II Response)

All reports of oil spills within the Coastal Marine Area of the
Bay of Plenty, are to be reported to:

Bay of Plenty Regional Council Pollution Hotline 0800 884 883

and

The Regional On-Scene Commander (ROSC)

Bay of Plenty Regional Council

Tauranga Office

ROSC e-mail	ROSC@boprc.govt.nz

ROSC Phone	027 405 8995

(Phone and email activated for significant spill lasting more than 24 hours)

Harbourmaster On-duty Officer	0800 5 KNOTS

ROSC1: Scott Robinson (to be appointed) 021 274 2472

ROSC2: Adrian Heays: 027 289 5020

Alternate ROSC

Adrian Heays	027 289 5020 (to be appointed)

Matt Harrex 	027 742 7349

John Morris	021 758 420

Derek Hartley	0800 800 401 (to be appointed) (WRC)

Salvatore (Sonny) Ali	(to be appointed) (GDC)

This Plan consists of two parts:

An operational section

- (The Operations Section guides the overall response)

A set of annexes

- (The Annexes contain supporting information that will assist with each stage of the response)

Important supporting documents

Emergency Operations Centre (EOC) Command Centre Resource Kit

National Marine Oil Spill Contingency Plan

Bay of Plenty Regional Council Regional Coastal Plan

New Zealand Nautical Almanac

Hydrographic Charts (In the chart table adjacent to the Maritime Team in Regional House)

New Zealand Tidal Streams Atlas – Maritime Team Library

Web EOC Library - Online

MSA’s Oil Spill Dispersants Guidelines

Prepared by:		Bay of Plenty Regional Council

Approved by:		Director Maritime New Zealand

Controlled Copy No: ______

Sections in this manual

Introduction and operations

Annex 1		Equipment and resources

Annex 2		Iwi engagement mobilisation and contacts

Annex 3		Communications

Annex 4		Sensitive areas

Annex 5		Prediction of oil movement

Annex 6		Document control

Annex 7		Glossary

Introduction and Operations

Contents

Introduction	9

Purpose of the Plan	9

Objectives of the regional marine oil spill response	9

Bay of Plenty Coastal Marine Area	10

Maritime New Zealand Web Emergency Operations Centre Database	11

Standard Operating Procedures	14

Phase One - Discovery, notification, evaluation,
identification, declaration, activation	14

Discovery and notification of a marine oil spill incident to
Bay of Plenty Regional Council	14

Evaluation and identification of a marine oil spill incident	15

No response required	15

No immediate response required - Standby Mode	15

Response required	15

Declaration – Tier 1, Tier 2 or Tier 3	16

Request for a Tier 3 response	16

Declaration of a Tier 2 response within an adjoining
region		19

Activation of Bay of Plenty Regional Council’s regional
marine oil spill responder group	20

Incident Management Team (IMT) and the Emergency Operations Centre (EOC)		20

General Incident Management Team Structure for a Tier 2 Response	21

Notification of interested parties	22

Phase Two - Development of a spill Incident Action Plan	23

IAP development team tasks	23

Phase Three - Incident Action Plan implementation	24

Safety		24

Regional On-Scene Commander and Incident Command
Centre Team financial authority	25

On-Scene Commander	25

Incident Command Team	25

Cost tracking and accounting	25

Media relations	25

Media releases during Tier 3 response operations	26

Equipment deployment	26

Documentation	26

Sampling and evidence collection	26

Security	26

Phase Four - Response termination and demobilisation	26

Transition from Response to Recovery and Termination	26

Equipment cleaning and demobilisation	27

Debriefing	27

Council reporting procedure	27

Phase Five - Post operations: Documentation of
costs/litigation	27

Policy		27

Financial systems	27

Links			28

Site Maps	32

Index			32

Introduction

Bay of Plenty Regional Council (BOPRC) has a statutory responsibility under the Maritime Transport Act 1994 to conduct a Tier 2 Response to marine oil spills that occur within the coastal marine area. A Tier 2 Response is an incident that is within the Regional Council’s capability to respond to. Accordingly, this Regional Spill Plan forms part of the New Zealand Marine Oil Spill Response Strategy and has been prepared in accordance with the Maritime Transport Act 1994, Rules and Amendments.

Purpose of the Plan

This Plan details how a Tier 2 Response operation is to be undertaken and should be referenced in response to a marine oil spill that occurs within the coastal marine area under the jurisdiction of
Bay of Plenty Regional Council.

Where spills relate to marine oil transfer sites, reference should also be made to the current and relevant Site Marine Oil Spill Contingency Plan (Site Plans) for the relevant Oil Transfer Site (OTS).

In the event of a Tier 3 Response in the Bay of Plenty, this Plan will provide MNZ with specific regional information to assist in responding effectively to the spill.

Objectives of the regional marine oil spill response

The primary objectives of this contingency Plan are to safely:

Prevent further pollution from the marine oil spill.

Contain and clean up the marine oil spill.

It must also be noted that in some spill situations, the spill will simply be monitored to ensure that no environmental damage occurs, and that no physical clean-up activities will be undertaken (e.g. diesel spill into a warm, rough sea).

Human health and safety have the highest priority in this Plan. The requirements of the Health and Safety at Work Act 2015 (HSWA) must be incorporated within any oil spill response undertaken.

Specific objectives are to safely:

Mobilise appropriate personnel and equipment in support of a Tier 2 Response operation anywhere in the Bay of Plenty’s coastal marine area.

Undertake appropriate containment, recovery and clean-up operations, where required, which mitigate the effects of the spilled oil and contribute to the restoration of the environment.

Initiate, when appropriate, wildlife deterrence, rescue and rehabilitation operations.

Complete clean-up operations as quickly and efficiently as available resources allow.

Minimise the extent of the impact as far as practical, taking into account ecological, physical, chemical, social, historical and cultural matters.

Gather evidence throughout the operation for possible legal action.

Maintain accurate records so that the cost of the operation can be accurately and continuously assessed.

Bay of Plenty Coastal Marine Area

A general description is the marine area extending twelve miles to seaward of the high water mark anywhere within the Bay of Plenty region. Broadly, the region is all that area of New Zealand from Potikirua Point in the east, to Orokawa Bay in the west. A more detailed outline of the Coastal Marine Area is contained in the Bay of Plenty Regional Coastal Environment Plan shown on the boundary map below:

Maritime New Zealand Web Emergency Operations Centre Database

Maritime New Zealand (MNZ) has a customised information management system called WebEOC (WebEOC) to assist with managing and sharing the information required by and generated during an incident response.

For example, WebEOC contains:

A database of National Response Team (NRT) and regional responders.

An equipment database.

Oil spill response standard operating procedures (SOPS), guidelines and advisories.

Position descriptions.

Systems for developing action plans during a response.

Systems for tracking costs during a response.

Systems for managing assets during a response.

The system is primarily designed for use during a national (Tier 3) response but is easily adapted for use during a regional (Tier 2) response. In order to use WebEOC during a Tier 2 response, contact Marine Pollution Response Service (MPRS) staff to set up an incident and provide staff logins. The system can also be used for exercises and training. Bay of Plenty Regional Council NRT staff will also already have access to WebEOC for incident response. In addition to use during a response, WebEOC is used to report all marine oil spills, to nominate responders for training, for planning exercises and managing the maintenance and deployment of MNZ equipment.

Standard Operating Procedures

Phase One - Discovery, notification, evaluation, identification, declaration, activation

Discovery and notification of a marine oil spill incident to
Bay of Plenty Regional Council

An oil spill report will require follow-up action to be taken in accordance with this Plan. All reports of oil spills within the Coastal Marine Area are to be directed to:

Bay of Plenty Regional Council POLLUTION HOTLINE 0800 884 883

Evaluation and identification of a marine oil spill incident

On receiving a report of an oil spill, the receiving officer shall:

Collect as much detail as possible by either completing the Pollution Incident Evaluation Form (WebEOC), or following normal complaint response procedures.

Investigate or arrange for an investigation to be immediately carried out in order to confirm the details surrounding the report.

Once verified notify the on-duty Regional On-Scene Commander (ROSC) by calling the
0800 5 KNOTS (0800 55 66 87) 24/7 number and provide all available information.

If the ROSC is unavailable, then contact an alternate OSC (On-Scene Commander). To do this, call the Duty Harbour Master on 0800 5 KNOTS (0800 55 66 87) and ask to be put in touch with the available OSC.

Contact the Group Manager of Regulatory Services or Chief Executive Officer (CEO) when an alternate On-Scene Commander (OSC) is required from outside BOPRC.

The ROSC will assess the information provided and decide upon the appropriate course of action with respect to a marine oil spill. These actions may include the following:

No response required

The ROSC will confirm that no response is required and notify spills and incidents via MNZs data management website: WebEOC.

No immediate response required - Standby Mode

If no immediate response is required (e.g. because oil is heading offshore), the ROSC will assess the situation and where necessary:

Put Incident Command Team on "standby" mode.

Advise RCCNZ via WebEOC or by phone 0508 472 269 (24 hours).

Establish monitoring of the slick and likely fate of the oil.

Be prepared to declare a response.

- Advise stakeholders as required.

Response required

If a response is required the ROSC will:

Determine whether the response should be part of a Regional (Tier 2) Response or a National (Tier 3) Response and make or request a declaration as appropriate (Refer to criteria in Section 2.7 and Escalation Matrix).

Advise RCCNZ by phone (0508 472 269) or notify via WebEOC.

Instruct the duty officer or nominee to initiate evidence collection (including obtaining samples of the spilled oil and photos). The enforcement section of the Compliance Team can assist and will generally lead an investigation in parallel to the response.

Advise stakeholders as required.

Declaration – Tier 1, Tier 2 or Tier 3

Tier 1 Oil Spill Response

A Tier 1 response is a spill that is within the capability of the OTS to respond to. Table 1 in Annex 4 – Sensitive Areas and Coastal Information, outlines the main oil transfer locations in the region. It shows the types of oil transferred, and expected order of spill magnitude. This is representative of the risk posed within the bunkering and bulk transfer stations in the Bay of Plenty. There are a number of bunker transfer sites located along the entire frontage of the Mount Maunganui Wharf at the Port of Tauranga. They are identified by the Port in terms of their location in metres from the northern end of the wharf as indicated on the map in Annex 4. Significant volumes of slops are also transferred from vessels at the container terminal at Sulphur Point, to tanker trucks. Details of all Oil Transfer Sites in the region, including current response plans may also be accessed in WebEOC.

The major oil industry sites have developed a port booming (Industry) plan with spill equipment maintained at the northern end of Mount Maunganui wharves at Berth 1 (on a spill trailer) and at the southern end of the wharves at Butters Landing. There is also a sorbent booming plan for the tanker terminal with industry spill equipment stored in storage boxes at the tanker berth. All major sites (tank farms) can initiate boom deployment. Bay Marine Works are the marine contractor for industry and can also deploy the equipment. Note there may be costs for deployment in cases where the spiller was not one of the major industry sites.

Diesel, lube and waste oil may be transferred via mobile plant from any of the wharves in the region and in the commercial port area. This includes refuelling activities in Whakatāne, on Lake Rotorua and the ferry at Ōmokoroa Wharf. These activities are considered a potential source of an oil spill. All transfer sites are required to have a current Transfer Site Spill Plan and a response capability relative to risk. This includes a ‘first strike’ containment capability and an on-water response to contain the estimated spill volume at the source. At the time of writing, there are 25 current Oil Transfer Site Plans that authorise oil transfers in the region.

The powers and authorities for marine oil spill response in this plan do not apply in freshwater environments except where there is a potential for the oil to enter the coastal marine environment. Permission to use MNZ equipment for freshwater spills is required and may be subject to charges for use.

Tier 2 criteria

An oil spill response will be declared a Tier 2 response in the following circumstances:

- The spill is within 12 nautical miles of land.

- Bay of Plenty Regional Council is able to respond adequately to the spill within the resources available.

- The response costs are within the capability and willingness of the Regional Council to pay. (The National On-Scene Commander (NOSC) can escalate a spill at any time).

- The spill exceeds the response capabilities of any Oil Transfer Site operator or for which no responsible party can be identified or is responsible.

If all of these criteria cannot be met, the ROSC (or any person authorised by them) should request the National On-Scene Commander to declare a Tier 3 response.

Request for a Tier 3 response

This request will be made via the Oil Spill Duty Officer (OSDO) phone 0508 472269 (24 hours). The ROSC should undertake such actions as are necessary to assist the National On-Scene Commander (NOSC) in accordance with the National Marine Oil Spill Contingency Plan.

Link to the National Marine Oil Spill Contingency Plan:

National Oil Spill Contingency Plan

The NOSC assume responsibility for managing a response if they consider it is beyond the capacity of Tier 2 resources, due to size, complexity or environmental impact.

Declaration of a Tier 2 response within an adjoining region

If the ROSC assesses the spill to be within Gisborne or Waikato regions area of responsibility, then they are to refer the report (and any relevant information) to the appropriate contact person within that region.

The contact telephone numbers are:

| Gisborne Unitary Authority | Reception Hotline Duty Officer ROSC ROSC AOSC | (06) 867 2049 027 652 7919 Salvatore “Sonny” Ali Mick Courtnell (MPRS) Adrian Heays |
| --- | --- | --- |
| Waikato Regional Council | Reception 24 Hours ROSC ROSC AOSC EMO Duty Officer | 0800 800 401 hold and ask for: Derek Hartley Kelly Hosking Adrian Heays Additional contact, EMO Duty Officer 0274 992 120 |

Activation of Bay of Plenty Regional Council’s regional marine oil spill responder group

The ROSC should initiate the following actions as appropriate, but not necessarily in this order. Maritime New Zealand should have already been contacted and appropriate sampling personnel dispatched in accordance with the “Response required” section above.

- Appoint personnel to monitor the spill/take initial actions to minimise the effects of the spill as are practical and appropriate.

- Arrange for investigation and sampling.

- Tier 1 operator should be notified and briefed as soon as possible with respect of transfer of responsibility.

- Establish the Emergency Operations Centre (EOC) as appropriate for the scale of the response.

- Designate personnel in each position who will assume the first roster.

- Activate required members of the response team and inform them of the location of the EOC and the time of briefings. Ensure that there is an adequate pool of personnel to allow for an ongoing response (i.e. greater than eight hours).

- Establish communications as required between the EOC and responders in the field.

- Notify stakeholders as appropriate.

- Arrange for preparation of media release.

- Ensure the chief executive, councillors and staff are kept informed.

Incident Management Team (IMT) and the Emergency Operations Centre (EOC)

In the early stages of an oil spill clean-up operation, it is likely that an interim communications base will be established in one of the responder’s vehicles or maritime operations vessels fitted with a cell phone and Marine VHF radios.

When a response requires significant coordination, the Incident Management Team will operate out of an EOC. The EOC is the centre of operations during the oil spill response. Where there is a need to establish an EOC, Emergency Management Bay of Plenty will lead the activation a facility under direction of the ROSC. The location and scale of the facility will be determined by the requirements of the response.

The positions and colour coding in the EOC for oil spill response adheres to the Co-ordinated Incident Management Structure (CIMS). The Co-ordinated Incident Management Structure establishes a framework of consistent principles, structures, functions, processes and terminology that agencies can apply in any emergency response. It is the primary reference for incident management in New Zealand.

The following diagram outlines the general Incident Management Team Structure for a Tier 2 Response.

General Incident Management Team Structure for a Tier 2 Response

Link for A2 print-out to use in an EOC - (Objective ID:A2605197) -
Response Structure PDF

During a large Tier 2 Spill, most positions in the team may be filled by one or more persons, given the need to ensure that any response operation occurs in eight hour shifts plus changeovers at most. If the response is relatively short term, shifts may be up to 12 hours plus change overs, at the ROSC’s discretion, having consideration for resourcing, knowledge transfer, effectiveness and health and safety.

This structure may change for smaller spills, as some positions may not be required. Alternatively, one person may be responsible for more than one of these roles.

Annex 1 – Equipment and Resources details equipment available in the EOC.

Annex 2 – Outlines personnel in the EOC and Regional Responder Team.

Legal Authority of the ROSC – The powers of a ROSC are set out in sections 300-328 of the Maritime Transport Act 1994. The roles and responsibilities of the ROSC, including financial authority, are given in Chapter 3 of the national plan (see the WebEOC Library and the Guidance for Regional Councils’ information shown in Annex 2).

Incident Management Team (IMT) (Planning/Operations/Logistics) – Once notified by the ROSC, the Logistics Manager will notify the other selected team managers, and selected team members and request they proceed to the EOC where they will be tasked with their duties.

Notification of interested parties

The ROSC, in consultation with the IMT, will determine which interested parties to contact and when to contact them (i.e. before or after the Response Action Plan is developed). It must be remembered that some, if not all the interested parties, will be able to provide some valuable input into development of the Response Action Plan.

Iwi and key stakeholder notifications: Key Iwi representatives and key stakeholders are to be notified via text message list at the initial declaration stage. The Duty Communications person will then notify the wider Iwi and key stakeholders as appropriate.

Notification must be carried out on a case by case basis depending upon the circumstances surrounding the spill.

It is the ultimate responsibility of the ROSC to ensure that all the interested parties are appropriately informed. What constitutes the interested parties will vary with each particular spill. Reference should be made to Annex 2 when determining which parties to notify.

As a general guide, other interested parties may include one or more of the following:

The Department of Conservation

Bay of Plenty District Health Board

Port of Tauranga

Ministry of Primary Industries

Local iwi and hapu groups (via iwi contacts tree maintained by Communications)

Heritage New Zealand – in case of archaeological discovery or to apply for an Archaeological Authority. In case skeletal remains are discovered, also notify the New Zealand Police.

Massey University/Waikato University

Marina owners and/or affected industry (check for industrial water takes in the affected area)

Local residents, commercial operators

Other wildlife groups (Forest and Bird, Project Jonah (marine mammals), Bird Rescue, Mauao Trust, etc).

District Councils/Neighbouring Regional Councils

Emergency Services (Police, Fire & Emergency New Zealand (FENZ) etc.)

Maritime New Zealand must be notified of every marine oil spill

- Any proposed wildlife response requiring the mobilisation of the national oiled wildlife response capability (Wildbase and/or Wildlife response equipment) shall be authorised by the OSDO or NOSC.

Phase Two - Development of a Spill Incident Action Plan

WebEOC provides templates for IAP’s, task plans and site safety plans.

The following order of events is typical of the procedure that will take place at the EOC with the arrival of the Incident Command Centre Team.

ROSC convenes a briefing meeting with the Incident Management Team (briefing forms are available in WebEOC).

ROSC forms an Incident Action Plan (IAP).

Each spill event requires an IAP tailored to the incident. The IAP sets out a clear strategy for spill responses, which is converted into an operational plan by the IMT. The draft IAP will normally be developed by the planning manager/team but for smaller responses may be done by the IMT.

The IAP Development Team must assess the appropriateness of any current spill response plan implemented at the Tier 1 level, and modify this as necessary.

IAP development team tasks

Evaluate the spill incident and any current spill Incident Action Plan. Annex 4 – Sensitive Areas and Coastal Information; (see the WebEOC Library and the Guidance for Regional Councils’ information shown in Annex 2) outlines clean-up and response options for different environments. In addition, the Regional Coastal Environment Plan contains information on Sensitive Areas and other Coastal Information (boprc.govt.nz/plans.) Annex 4 is to be used in the first instance.

Determine both short term and long term objectives of the response.

Development of the IAP should include:

- the strategy for the response and necessary actions to be undertaken, including a strategy for wildlife rescue and rehabilitation if required. Any proposed wildlife response requiring the mobilisation of the National Oiled Wildlife Response capability (Wildbase and or Wildlife response equipment) shall be authorised by the OSDO or NOSC,

- clear aims and objectives for all actions,

- clear timeline for all actions to achieve objectives, and

- clear statement of responsibility (i.e. which members of the team are responsible) for the actions and tasks.

Determine the resources and expertise needed, and those available. Annex 1 and 2 provide equipment and personnel lists along with mobilisation instructions.

Provide a mechanism for feedback, with continuous monitoring of the spill response and modification of the IAP as appropriate.

The ROSC will communicate the IAP to the entire Incident Command Centre Team, and is responsible for ensuring that the OSDO or NOSC is also briefed of the action.

Phase Three - Incident Action Plan implementation

Safety

The safety of human life is to take precedence over all aspects of the response operation. Persons employed in the response operation are to comply with the Health and Safety at Work Act 2015. In the event of a response requiring a significant number of responders, it is advisable to introduce a Health and Safety Advisor in the response team to manage this area. See WebEOC for more information including Health and Safety procedures and protocols.

A Health and Safety Advisor is also a specialist role in the NRT and MNZ may be able to provide a coordinator from the NRT. Depending on the nature of the spill, an odour specialist may also be used to assess the health risks to responders who may be being exposed to volatile organic compounds (VOCs).

Also refer to the Bay of Plenty Regional Council Health and Safety Policy and procedures for guidance.

Regional On-Scene Commander and Incident Command Centre Team financial authority

On-Scene Commander

What follows is a guide on all expenditure of up to $250,000 with the following provisos:

- That the Chief Executive approves the delegation to the Regional On-Scene Commander to spend a maximum of $100,000 (inclusive of GST) for any one event without further reference. If the expenditure exceeds or is likely to exceed $100,000, the Regional On-Scene Commander will seek approval from their General Manager and in their absence, the Chief Executive.

- That the Chief Executive approves the delegation to the alternate Regional On-Scene Commanders to spend a maximum of $50,000, (inclusive of GST) for any one event without further reference. If expenditure exceeds or is likely to exceed $50,000, then the alternate On-Scene Commander will seek approval from the Regional On-Scene Commander who may approve expenditures up to $100,000. For expenditure beyond $100,000, the Regional On-Scene Commander will seek approval from the General Manager and in their absence, the Chief Executive. Should the Regional On-Scene Commander not be available, approvals above $50,000 should be sought from the General Manager/Chief Executive.

- In the circumstances of an extreme emergency, where the approval process set out above cannot be followed because of either the difficulties in communications or the unavailability of all of those in the approval chain, then all parties should act in good faith. They should make the decisions that they feel are appropriate at the time, reporting on the decision that they have made at the earliest possible date following the approval process set out in 1 and 2 above.

Incident Command Team

The ROSC can delegate expenditure limits up to $100,000.00 as is appropriate for the efficiency of the response. The ROSC needs to instruct Finance to make the delegation operational in the event that the funds are required. The ROSC delegation is set and authorised in the Regional Council’s Delegations Manual.

The Incident Command Team has, in general, an overall limit on all expenditure of $5,000. Any expenditure over the above limits must be authorised by the Regional On-Scene Commander.

Cost tracking and accounting

Cost tracking must be rigorously applied throughout the response. WebEOC contains guidance on the financial procedures that must be followed during an oil spill response. High priority must be given to the gathering of sufficient and accurate information to enable recovery of costs from the spiller.

A finance recording spreadsheet template can be found here:

Objective Link: (A2629706) Oil Spill Finance Tracking.

Objective Link: (A3488100) 2020 BOPRC Boat Charge-out rates

Media relations

Co-operative media relations must be developed early in the response and regular press releases made during the response (by the Public Information Manager (PIM)) are to be approved by the ROSC. Unless otherwise approved by ROSC, the only people in the Response Team who will communicate with the media during an oil spill response will be the ROSC and the Media Liaison Advisor.

Refer to the Maritime New Zealand: National Oil Spill Contingency Plan

Also refer to guidance for regional councils in WebEOC

Incident Command Roles and Responsibilities

Media and Community Relations

The Media Liaison Advisor should forward media queries outside the subject of the spill response to the appropriate organisation or individual to provide that comment.

Media releases during Tier 3 response operations

Releases of information regarding a Tier 3 Response operation to the media are to be made only with the authority of the Director, MNZ, or the National On-Scene Commander. Regional responders in a Tier 3 operation are to decline comment with the media and refer all enquires to the aforementioned persons.

Equipment deployment

Accurate records of equipment dispatch as well as the use/disposal of disposable items must be maintained. All dispatch should be channelled through a controlled area using manual or electronic dispatch and return forms. A procurement/dispatch officer may be appointed at the outset of a large spill.

Documentation

Records of all communications (telephone conversations, emails and file notes must be recorded), all financial transactions and expenditure, and a chronological account of the incident must be kept.

Sampling and evidence collection

Refer to the Maritime New Zealand National Oil Spill Contingency Plan Guidance for Regional Councils in WebEOC:

Refer to MNZ Guidelines for Regional Councils – Prosecution and Cost Recovery, located in the WebEOC file library.

Security

Security for the Incident Command Centre, and the response operation in the field must be installed for the safety of response personnel and the public, protection of equipment and maintaining accessibility to those area affected by the spill.

Phase Four - Response termination and demobilisation

Transition from Response to Recovery and Termination

The ROSC may terminate any marine oil spill response by Bay of Plenty Regional Council (Section 304 (2) MTA 1994). The decision should be made on reaching all objectives and agreed transition points.

Prior to seeking termination of the response, the ROSC will hold a meeting with the Incident Command Centre Team. The purpose of this meeting is to determine whether the IAP objectives have been achieved and the incident response has been adequately completed.

If this decision is likely to be contentious, then the decision may be referred to a National On-Scene Commander or Director, Maritime New Zealand, for resolution.

Response termination involves the recovery, cleaning and maintenance of all equipment used during the response, as well as the demobilisation of all personnel involved in the response and the replacement of consumables. Also, it involves the collation and completion of all documentation associated with the spill response, including expenditure reports.

The recovery phase consists of the period of time following the completion of physical oil containment and removal that the activities such as environmental and cultural monitoring related to the incident continue.

Equipment cleaning and demobilisation

The cleaning of response equipment used during a response will be carried out in such a way as to not cause further contamination of other areas or sites. Cleaning should be carried out in a contained area where oil and contaminant residues can be contained for final disposal.

All renewable equipment should be thoroughly inspected for damage and maintenance prior to being cleaned/repacked for storage. Accurate demobilisation records must be kept.

Debriefing

A debriefing of key personnel will be held following termination of the response. This will enable a review of the appropriate Plans and highlight areas where the response and planning could be improved.

The ROSC is responsible for arranging the time and venue of the debriefing and shall inform those persons/or representatives of supporting organisations of such arrangements. Those persons and/or representatives are expected to attend the debriefing. Costs associated with attending the debriefing or the completion of reports shall be considered to be part of the overall incident response.

Council reporting procedure

For oil spills with costs in excess of $10,000.00

On completion of the debriefing, the ROSC is to ensure that all pertinent information is collated and forwarded together with a report to the Chief Executive, Bay of Plenty Regional Council, within 20 working days of the debriefing.

If no response is required, then a brief report should be drafted for inclusion in the maritime report to Council. A more comprehensive report is required if clean-up operations are undertaken, particularly if cost recovery is to be undertaken.

Phase Five - Post operations: Documentation of costs/litigation

Policy

It is the policy of Bay of Plenty Regional Council, and MNZ, to recover the costs of marine oil pollution clean-up operations from the spiller.

Financial systems

Refer to the Maritime New Zealand – National oil Spill Contingency Plan and supporting documents in WebEOC. In particular the Guidelines for Regional Councils found in the Library Section of WebEOC.

Also Objective Link: (A2629706) Oil Spill Finance Tracking

It must be noted that costs will still be incurred after the termination of the clean-up phase of the incident and these need to be accounted for in the overall response cost.

For information and costing related to environmental and baseline monitoring and analysis please see:

Objective Link: (A2956616) 2018-09-05 Costs and Process for Environmental Monitoring

Links

| Introduction and Operations |  |
| --- | --- |
| Objective ID:A2605197 For A2 print-out for use in an EOC | Response Structure PDF |
| Maritime New Zealand National Oil Spill Contingency Plan | National Oil Spill Contingency Plan |
| Objective Link: A2629706 | Oil Spill Finance Tracking |
| Objective Link: A3718750 | Collection of oiled wildlife for evidence |
| Objective Link: A3749135 | Confidential Contacts for marine oil spill response |

| Annex 1 |  |
| --- | --- |
| Objective Link: A2749992 | 2017-11-24 Sign-in/site induction/timesheets |
| Objective Link: A2692405 | 2017-09-05 Site Supervisors Induction Checklist |
| Objective Link: A2860162 | A Skippers Industry Specific Training Program Spreadsheet (This file is protected – please see a BOPRC Maritime staff member to open) |
| Objective Link: A2216535 | 2015-11-06 Oil Spill Response Inventory |
| For a link to a folder containing information on vessel fleet operating limits, charge-out rates and all vessel manuals | Objective Link (folder): (fA178077) |
| Regional On-Site Commander email | ROSC@boprc.govt.nz |
| MNZ Equipment and Contract Objective link: A2501436 The right obj.Id is: A3529046 | 2019-12-04 Recent changes to the oil spill equipment held by BOPRC |
| Intergroup Liquid/Solid Waste Objective link: A2552915 | 2017-02-22 Intergroup Price and Services Guide |
| EnviroNZ Services MNZ Contract Objective Link: A2627864 | ESL-MNZ Final Service Contract - valid until 2022 |
| EnviroNZ Price Lists Objective Link: A2704309 | 2020-11-05 ESL Price List - 1 November 2020 |
| Pacific Dive Information Objective Link: A2717121 | 2017-10-12 Glenn Ladkin Pacific Diving Capability, Services and Pricing |
| Diving and Salvage Contractors Pacific Diving Bay Underwater Services New Zealand Limited Greenfields Diving Services | www.pacificdiving.co.nz www.bayunderwater.co.nz http://greenfielddiving.co.nz/ |

| Objective Link: A2658058 | 2020-11-11 Updated & Suitable Oiled Wildlife Recovery Site Information |
| --- | --- |
| Glencoe Marine | www.glencoemarine.co.nz |
| Objective Link: A27783 Correct obj id is: A3712788 | Major Incident Air Shelter – See the user’s manual – Link needs updating |
| Objective link: A2277509 | Guidance for the Appointment and Development of Oil Spill Responders |
| Objective Link: fA178077 | BOPRC Boat Fleet Folder – Not correct. Delete and write: Objective folder only – no link available |
| delete link – out of date and is correct on the link above | – Delete link |
| Objective Link: A2682071 | Awanui's Operating Manual Sections 3.11-3.10 |
| Objective Link: fA959926 | 2017-11-24 Final Approved Industry Plan - Objective folder only – no link available |
| Objective Link: A3749135 | Confidential contacts for marine oil spill response.docx |
| Objective Link: A3749136 | Care Group Leader Contact Details can you make a link? |
| Objective Link: A3816908 | Korimako Customer Information May 2021 |
| Objective Link: A3816907 | Albatros Dredge at Port of Tauranga |
| Objective Link: A3816911 | Albatros Skimmer System Koseq Compact |

| Annex 2 |  |
| --- | --- |
| Objective ID: A2763843 | Regional Responders List |
| Objective Link: A2600582 | Maritime Duty Harbourmaster Contacts |
| Bay of Plenty Regional Council | https://www.boprc.govt.nz/about-council/kaupapa-maori/hapuiwi-resource-management-plans/ |
| Objective Link: A3749135 | Confidential Contacts List |

| Annex 3 |  |
| --- | --- |
| For vessel information Objective Link: (A2546762) | 2017-02-14 Hand-Held Radio Specifications |
| Spark 2Degrees Vodafone | http://www.spark.co.nz/coverage http://www.2degreesmobile.co.nz/coverage; https://www.vodafone.co.nz/network/coverage/ |

| Annex 4 |  |
| --- | --- |
| Objective folder link ID: fA959926 | Port/Industry Booming Plan - - Objective folder only – no link available |
| Health and Safety Induction | www.port-tauranga.co.nz/health-safety/port-inductions/ |
| IUCN Classification Scheme | www.iucnredlist.org |
|  | http://www.moutohora.co.nz/ www.volcanicair.co.nz |
| Objective ID: fA342204 | Site Maps |
| White Island general information | www.wi.co.nz Delete – link does not work |
|  | www.geonet.org.nz. |
|  | www.nzbirds.com/birds/takapu.html |
|  | mandyart.blogspot.com/2011/02/seal-census-white-island.html |
|  | nzbirds.com/birds/greyternlet.html |
|  | mandyart.blogspot.com/2011/02/seal-census-white-island.html duplicate – delete |
| Objective Link: zA200611 | Waste Management FINAL Approved Tier 1 Plans |
| Bay of Plenty Regional Council Coastal Plan | https://www.boprc.govt.nz/your-council/plans-and-policies/plans/regional-plans/regional-coastal-environment-plan |
| Objective ID: A3703087 | Notated Bunker Line lateral layout Mt Maunganui – 2020-12-15 – link Is not working |
| Objective ID: A3703051 | 2020-12-12 PoT Updated Stormwater Lines for Mt Maunganui 337-05 – link not working |
| Objective Links: fA1265038 and A3611516 | Waste Management Oil Recovery FINAL Approved OTS Plan and signoffs Waste Management Technical Services FINAL Approved OTS Plan and signoffs (This text is not a link to the document – please open from Objective at the above reference). |
| Objective link: A3718750 | Guide for the collection of oiled wildlife for evidence |
| Objective link: ID A3752596 | End-point criteria for bulk oil removal and site sign-off procedures |
| Objective link: ID A3816904 | Procedure for managing archaeological sites |

| Annex 5 |  |
| --- | --- |
| Land Information New Zealand Current predictions, tide tables and current information | www.linz.govt.nz/hydro |
| MetService | www.metservice.co.nz |
| Port of Tauranga Wind, tide and current information | www.port-tauranga.co.nz/Harbour-Conditions |

| Annex 6 |  |
| --- | --- |
| Bay of Plenty Regional Council Plans and policies. Note: This is an intranet link available only to BOPRC staff | http://sam.envbop.net/working-here/resources/procedures-and-guidelines/ |
| Regional On-Scene Commander - BOPRC | ROSC@boprc.govt.nz. |

Site Maps

Index

Site Maps available in Objective – Folder ID fA342204

| Site No. | Description |
| --- | --- |
|  | Index Map |
|  | Port of Tauranga – Sulphur Point |
|  | Port of Tauranga – Mt Maunganui |
| 01 | Waiau/Athenree |
| 02 | North Tanners Point |
| 03 | Upper Tuapiro Estuary |
| 04 | Tuapiro Estuary Spit |
| 05 | Uretara Upper |
| 06 | Uretara East |
| 07 | Waikoura Point |
| 08 | Rereatukahia |
| 09 | Puketutu Point |
| 10 | Matakana North |
| 11 | Maheka Point |
| 12 | Matakana South |
| 13 | Aongatete |
| 14 | Wainui |
| 15 | Central Mangroves |
| 16 | Te Hopai Island |
| 17 | Apata Estuary |
| 18 | Blue Gum Bay |
| 19 | Pahoia North |
| 20 | Pahoia South |
| 21 | Tahunamanu Spit/Island |
| 22 | Hunter Creek |
| 23 | Opureora Spit |
| 24 | Motungaio Island |
| 25 | Te Puna |
| 26 | Rangiwāea East |
| 27 | Rangiwāea West Estuary |
| 28 | Panepane Point |
| 29 | Wairoa River |
| 30 | Matua |
| 31 | Mauao – Mount Maunganui |
| 32 | Moturiki (Leisure) Island |
| 33 | Motuotau (Rabbit) Island |
| 34 | Waikareao Estuary |
| 35 | Waimapu – Yatton Park |
| 36 | Waimapu – Windermere |
| 37 | Waipu Bay/Tauranga Airport |
| 38 | Rangataua Bay North |
| 39 | Welcome Bay |
| 40 | Rangataua Bay South |
| 41 | Motiti Island |
| 42 | Taumaihi Island (‘The Knoll’) |
| 43 | Maketū Estuary, beach and spit |
| 44 | Ōkurei Point |
| 45 | Little Waihī Estuary |
| 46 | Waitahanui Stream |
| 47 | Hauone Stream |
| 48 | Pikowai Stream |
| 49 | Herepuru Stream |
| 50 | Matatā Lagoon |
| 51 | Rangitāiki River East |
| 52 | Whakatāne River/Estuary |
| 53 | Kōhī Point |
| 54 | Ōhiwa Harbour/Estuary |
| 55 | Waiotahi Estuary |
| 56 | Waioeka Estuary |
| 57 | Waiaua Estuary |
| 58 | Waiiti River and Tōrere |
| 59 | Mōtū River |
| 60 | Haparapara |
| 61 | Kereru River Mouth |
| 62 | Raukōkore River |
| 63 | Orete Point |
| 64 | Oruatiti and Waikanapanapa |
| 65 | Whangaparaoa River |
| 66 | Otarawhata Island |
| 67 | Whakaari (White Island/Volkner Rocks) |
| 68 | Karewa Island |
| 69 | Tūhua (Mayor Island) |
| 70 | Moutoki Island |
| 71 | Rūrima Island |
| 72 | Motunau (Plate) Island |
| 73 | Motuhora (Whale) Island |
| 74 | Motuhaku (Schooner Rocks) |